# CERT Emergency Communications Handout: Options for Grid-Up and Grid-Down Situations

## Preparing Your Communication Strategy

Before selecting tools, clarify your objectives with these key questions to define your mission:

- Who do you want to talk to? (e.g., family nearby, CERT team across town, emergency services, or distant relatives?)

- What information do you need to give them? (e.g., simple status check, detailed location/GPS, voice report, or data like photos/maps?)

- How far away are they? (e.g., within a neighborhood (1-5 miles), regional (10-50 miles), national, or global?)

- Is the grid up or down? (e.g., partial outage with some cell/internet working, or total blackout with no power/internet?)

These answers will guide your choices. In grid-up scenarios (power/cell/internet partially available), prioritize fast, app-based tools. In grid-down (total failure), focus on battery/solar-powered, infrastructure-independent options. Always train, test, and include spares in your go-bag. Hybrid use: Start with grid-up tools, fallback to grid-down.

## Communication Options by Scenario and Category

Options are grouped by scenario, then category (local to global). Each includes key features, approximate costs (2025 USD), and pros/cons. Mix and match based on your answers—e.g., cell apps for grid-up locals, mesh for grid-down teams.

### Grid-Up Situations (Infrastructure Available: Cell Towers/Power/Internet Partial)

Leverage existing networks for quick, high-bandwidth comms. Best for early response when services are spotty but functional.

#### 1. Cell Phones & Apps (Local/Regional, 1-50+ Miles)

Standard smartphones with data; use for voice/text/data if signal holds.

| Device/App | Key Features | Price | Battery Life | Pros | Cons |
| --- | --- | --- | --- | --- | --- |
| Smartphone (e.g., iPhone/Android w/ Carrier Plan) | Voice calls; SMS; GPS sharing; apps like WhatApp/Facebook Messenger | $500+ (device) + $30-60/mo plan | 10-20 hours (use w/power bank) | Ubiquitous; group chats/video; maps integration | Signal-dependent; drains battery fast |
| Zello Walkie-Talkie App | Push-to-talk over data; channels for CERT groups; offline caching | Free (app) | Phone-dependent | Instant team voice like radios; global if WiFi | Needs data/WiFi; no good in full outage |
| Nextdoor or Citizen Apps | Neighborhood alerts; real-time incident reports; location-based | Free | Phone-dependent | Community coordination; integrates w/emergency services | Urban-focused; privacy concerns |

Best For: Initial alerts or family check-ins while towers are up. Enable low-data mode; charge via car/solar.

#### 2. Cell Phone Plans with Satellite Messaging (Regional/Global Hybrid, 10+ Miles)

Enhances standard cell plans with satellite fallback for texts/location when towers fail. Works on compatible phones (no extra hardware); ideal for grid-up transitions to spotty coverage. Requires clear sky view; messages may delay 1-5 min.

| Carrier | Key Features | Monthly Cost | Compatible Devices | Coverage | Pros | Cons |
| --- | --- | --- | --- | --- | --- | --- |
| T-Mobile (T-Satellite via Starlink) | Texting, location sharing, app access (e.g., WhatsApp, Maps); picture/data rollout; auto-connect | Included on Go5G Next/Experience Beyond ($90+/mo plans); $10/mo add-on for others | 60+ iOS/Android (e.g., iPhone 14+, Galaxy S24+) | Cont. US, PR, HI, parts AK (500K+ sq mi dead zones) | Seamless integration; optimized low-bandwidth apps | Delays in obstructed areas; promo pricing may end |
| AT&T (via AST SpaceMobile/Starlink) | Voice, text, data, video; peer-to-peer/emergency messaging; push-to-talk for first responders | Included for ~50% of customers; $15/mo add-on for Starlink texting on others | iPhone 14+, Google Pixel 8+ | Cont. US (100% nationwide goal, incl. indoors) | Broad device support; public safety focus | Costs not uniform; rollout ongoing |
| Verizon (Messages via Satellite) | Texting, location/emergency comms; future group/photo/video; works in airplane mode | Included with myPlan satellite service (on Unlimited Ultimate/Plus, $80+/mo); no extra cost | iPhone 14+ (iOS 18), Samsung Galaxy S25/Z Fold7/Flip7, Pixel 9+ | Cont. US, HI, parts AK | Auto-detection; integrates w/native apps | Line-of-sight needed; limited future features now |
| Google Fi (Satellite SOS/Messaging) | Emergency SOS texting/location; iMessage satellite on iPhones; WhatsApp calls on Pixels | Included on Unlimited Premium ($65/mo); no add-on fee | Pixel 9+ (full), iPhone 16+ (iMessage); limited non-emergency | Cont. US (emergency focus) | Simple for Fi users; Pixel integration | Mostly emergency; no full texting on all devices |

Best For: Extending cell coverage to remote/rural CERT ops (e.g., texting HQ from trailheads). Check carrier eligibility; test in low-signal areas. Apple's iMessage satellite works cross-carrier for iPhone users.

#### 3. Mesh Networks (Local Teams, 1-5 Miles; Extends w/o Towers)

App-based peer-to-peer via Bluetooth/WiFi; creates local nets if cell fades.

| Device/App | Key Features | Price | Battery Life | Pros | Cons |
| --- | --- | --- | --- | --- | --- |
| goTenna Mesh | Text/voice/GPS via paired devices; offloads to mesh if cell drops | $180 (pair) | 24+ hours (phone-dependent) | Builds ad-hoc nets (up to 4 miles/device); no sub needed | Limited to app users; range drops in buildings |
| Beartooth App + Device | Offline texting/SOS; pairs w/smartphone; group channels | $130 (device) | 48 hours | Simple setup; works w/any phone; emergency broadcast | iOS/Android only; 2-3 mile urban range |
| Bridgefy App | Bluetooth mesh texting; no internet; group chats up to 100m direct | Free (app) | Phone-dependent | Instant in crowds (e.g., evacuations); scalable | Short range without relays; Android/iOS compatibility issues |

Best For: CERT team huddles in partial outages. Pair with cell for hybrid—auto-fails to mesh.

#### 4. Broadband & Internet Tools (Regional/Global, Any Distance)

High-speed data for maps/video if internet lingers.

| Device | Key Features | Price | Power/Battery | Pros | Cons |
| --- | --- | --- | --- | --- | --- |
| Starlink Mini Kit | Portable dish; 50-100Mbps; app for remote control | $599 (kit) + $50/mo | 25-40W (USB-C; solar ok) | Fast video/maps; global if sky clear | Power-hungry; setup needs elevation |

Best For: Command posts sharing intel. Use mobile hotspot mode.

### Grid-Down Situations (Total Blackout: No Power/Cell/Internet)

Infrastructure-independent; solar/battery focus for prolonged ops.

#### 1. Local Tactical Radios (Neighborhood/Team, 1-5 Miles)

No licenses for basics; simplex mode for direct links.

| Device | Key Features | Price | Battery Life | Pros | Cons |
| --- | --- | --- | --- | --- | --- |
| Midland GXT1000VP4 (FRS/GMRS) | 36-mile claimed; NOAA; privacy codes; waterproof | $80 (pair) | 10-15 hours | Group channels; hands-free | GMRS license ($35) for full power |
| Retevis RT22 (FRS) | 16 channels; compact; VOX | $20-30 | 12 hours | License-free; pocketable | 2-mile urban max |

Best For: Door-to-door. Pre-program simplex channels.

#### 2. Ham Radio Gear (Regional Voice, 10-50+ Miles)

FCC Technician license needed; add elevation for reach.

| Device/Accessory | Key Features | Price | Battery Life/Power | Pros | Cons |
| --- | --- | --- | --- | --- | --- |
| Baofeng UV-5R (VHF/UHF) | Dual-band; 5W; CERT nets | $25-35 | 8-12 hours | Affordable; simplex | Range needs height |
| Solar Power Kit (Goal Zero Nomad 20 + Jackery 240) | 20W panel + 240Wh battery | $200-300 | 4-6 hours charge | Powers all gear | Bulky; sun-dependent |

Best For: Linking to external aid. Use 146.520 MHz.

#### 3. Digital Modes & Enhancements (Data/Email Over Radio)

Text/email via radio waves; no net required.

| Device/Software | Key Features | Price | Battery/Compatibility | Pros | Cons |
| --- | --- | --- | --- | --- | --- |
| Winlink (RMS Express) | HF/VHF email; VARA protocol | Free | Radio-dependent | Structured reports | Interface needed |
| WoAD App (Android) | Mobile Winlink; USB radio link | €7 | Phone battery | Portable data | Setup curve |
| DMR Hotspot + Radios (AnyTone AT-D878UV + Pi-Star) | Digital voice/data; talkgroups | $400 total | 12 hours | Clear, efficient | Power for hotspot |

Best For: Detailed updates (e.g., ICS forms). Test VARA.

#### 4. Satellite Messengers/Phones (Global, Any Distance)

Sky-view only; subscriptions for airtime.

| Device | Key Features | Price | Battery Life | Pros | Cons |
| --- | --- | --- | --- | --- | --- |
| Inmarsat IsatPhone 2 | Voice/SMS; SOS/GPS | $789 | 8 hours talk | Budget phone | Bulkier |
| ACR Bivy Stick (MESH) | Texting; LoRa relays; groups | $300 | 145 hours | Local mesh bonus | App needs phone |
| Garmin inReach Messenger Plus | Unlimited texts; weather/SOS | $400 | 28 days | Maps/tracking | $15+/mo |
| ZOLEO Communicator | Texting; check-ins | $200 | 200 hours | Group-friendly | Slower |
| Iridium 9555 | Voice/SMS; hotspot | $1,200 | 4 hours talk | Rugged standalone | Heavy/pricey |
| SPOT X | Texting; GPS groups | $250 | 240 hours | Basic no-sub | Limited chars |

Best For: External contact. Annual plans for CERT.

#### 5. Mesh Networks (Local/Global Hybrid, 1-5+ Miles)

Peer-to-peer for teams; satellite/mesh combos.

| Device/App | Key Features | Price | Battery Life | Pros | Cons |
| --- | --- | --- | --- | --- | --- |
| goTenna Mesh | Text/voice; relays build net | $180 (pair) | 24+ hours | Off-grid extension | User-limited |
| Beartooth | Offline SOS/text | $130 | 48 hours | Broadcast mode | Short direct range |
| Bridgefy App | Bluetooth groups | Free | Phone-dependent | Crowd-scale | Relay-dependent |

Best For: Decentralized teams in blackouts.

## Final Tips for CERT Success

- Hybrid Strategy: Grid-up: Cell/satellite plans first. Grid-down: Radios/sat fallback. Power: Solar essentials for all.

- Training: License-free start (FRS/mesh); add ham/sat. Weekly drills.

- Kit Cost: $50-200/person basics; $500+ advanced.

- Resources: FCC.gov licenses; ARRL.org guides; local CERT.